# STM32G474RET6 — Dual-Axis Controller PCB

## Pin Assignments & Functionality  (Rev 7 — LQFP64)

LQFP-64  ·  Dark red = new/reassigned  ·  Dark green = unused/available  ·  Grey = power/dedicated

| Pin # | GPIO | Signal Label | Peripheral / Function | Dir | Pull | Description |
| --- | --- | --- | --- | --- | --- | --- |
| RS-485 — HOST PORT (FOLLOWER / MODBUS SLAVE) · Top side, pins 57-59 |  |  |  |  |  |  |
| 57 | PB5 | RS485_HOST_DE | GPIO_Output | OUT | — | Driver enable for host RS-485 transceiver (HIGH=transmit) |
| 58 | PB6 | USART1_TX | USART1 | OUT | — | UART TX → host RS-485 transceiver — Modbus RTU to host controller |
| 59 | PB7 | USART1_RX | USART1 | IN | — | UART RX ← host RS-485 transceiver — Modbus RTU from host |
| RS-485 — LEADER PORT (OUTGOING / FUTURE SERVO CONTROLLER) · Top side, pins 51-52 & 55 |  |  |  |  |  |  |
| 51 | PC10 | USART3_TX | USART3 | OUT | — | UART TX → leader RS-485 transceiver — Modbus RTU to downstream device |
| 52 | PC11 | USART3_RX | USART3 | IN | — | UART RX ← leader RS-485 transceiver — response from downstream device |
| 55 | PD2 | RS485_LEAD_DE | GPIO_Output | OUT | — | Driver enable for leader RS-485 transceiver (HIGH=transmit) |
| RS-232 PORT (GPS NMEA via MAX232) · LPUART1: TX=PB11 (right, pin 33), RX=PB10 (bottom, pin 30) |  |  |  |  |  |  |
| 33 | PB11 | LPUART1_TX | LPUART1 | OUT | — | LPUART1 TX → MAX232 level shifter — RS-232 outgoing data |
| 30 | PB10 | LPUART1_RX | LPUART1 | IN | — | LPUART1 RX ← MAX232 — GPS NMEA sentence input |
| GPS 1 PPS TIMING INPUT · Top side, pin 61 |  |  |  |  |  |  |
| 61 | PB9 | GPS_1PPS | EXTI9 / TIM input | IN | PULLDOWN | 1 Hz precision timing pulse from GPS — sidereal sync / RTC calibration |
| SPI2 — W25Q32 4 MB NOR FLASH · Right side, pins 34-37 (contiguous PB12-PB15) |  |  |  |  |  |  |
| 34 | PB12 | FLASH_CS | GPIO_Output | OUT | — | SPI chip-select for W25Q32 (active LOW) |
| 35 | PB13 | SPI2_SCK | SPI2 | OUT | — | SPI clock to W25Q32 |
| 36 | PB14 | SPI2_MISO | SPI2 | IN | — | SPI MISO from W25Q32 |
| 37 | PB15 | SPI2_MOSI | SPI2 | OUT | — | SPI MOSI to W25Q32 |
| I2C4 — SENSOR BUS · Right side, pins 38-39 [I2C4 replaces I2C3; PC6=SCL, PC7=SDA] |  |  |  |  |  |  |
| 38 | PC6 | I2C4_SCL | I2C4 | OUT | — | I2C clock — on-board sensors: LM75B temp, LTC4311 extender, M24C64 EEPROM |
| 39 | PC7 | I2C4_SDA | I2C4 | I/O | — | I2C data — bidirectional sensor bus |
| AXIS 1 — ENCODER (E6B2-CWZ6C) · Left side, pins 12-14 |  |  |  |  |  |  |
| 12 | PA0 | ENC_A | TIM2_CH1 | IN | — | Encoder channel A — quadrature input (TIM2 encoder mode) |
| 13 | PA1 | ENC_B | TIM2_CH2 | IN | — | Encoder channel B — quadrature input (TIM2 encoder mode) |
| 14 | PA2 | ENC_Z | EXTI2 | IN | PULLUP | Encoder index/Z pulse — external interrupt for homing reference |
| AXIS 1 — DM556Y STEPPER DRIVER · Bottom side, pins 24-26 |  |  |  |  |  |  |
| 24 | PB0 | PUL | TIM3_CH3 PWM | OUT | — | Step pulse to DM556Y PUL+ (variable-frequency PWM for speed control) |
| 25 | PB1 | DIR | GPIO_Output | OUT | — | Direction signal to DM556Y DIR+ (HIGH=CW, LOW=CCW) |
| 26 | PB2 | ENABLE | GPIO_Output | OUT | — | Motor driver enable — assert to energize motor coils |
| AXIS 1 — LIMIT SWITCHES · Bottom side, pins 17-18 |  |  |  |  |  |  |
| 17 | PA3 | LIMIT_SW1 | GPIO_Input | IN | PULLUP | Axis 1 limit switch 1 — active LOW (normally open, pulled high) |
| 18 | PA4 | LIMIT_SW2 | GPIO_Input | IN | PULLUP | Axis 1 limit switch 2 — active LOW (normally open, pulled high) |
| AXIS 2 — ENCODER (E6B2-CWZ6C) · Left side, pins 8-10 [NEW — TIM1 encoder mode] |  |  |  |  |  |  |
| 8 | PC0 | ENC_A2 | TIM1_CH1 | IN | — | Encoder 2 channel A — quadrature input (TIM1 encoder mode) |
| 9 | PC1 | ENC_B2 | TIM1_CH2 | IN | — | Encoder 2 channel B — quadrature input (TIM1 encoder mode) |
| 10 | PC2 | ENC_Z2 | EXTI10 | IN | PULLUP | Encoder 2 index/Z pulse — external interrupt for homing reference |
| AXIS 2 — DM556Y STEPPER DRIVER · Right side pins 40-41, bottom pin 19 [NEW — TIM8 PWM] |  |  |  |  |  |  |
| 40 | PC8 | PUL2 | TIM8_CH3 PWM | OUT | — | Step pulse 2 to DM556Y PUL+ (TIM8_CH3 variable-frequency PWM) |
| 41 | PC9 | DIR2 | GPIO_Output | OUT | — | Direction 2 to DM556Y DIR+ (HIGH=CW, LOW=CCW) |
| 19 | PA5 | ENABLE2 | GPIO_Output | OUT | — | Motor driver 2 enable — assert to energize motor coils |
| AXIS 2 — LIMIT SWITCHES · Bottom side, pins 20-21 [NEW] |  |  |  |  |  |  |
| 20 | PA6 | LIMIT_SW3 | GPIO_Input | IN | PULLUP | Axis 2 limit switch 1 — active LOW (normally open, pulled high) |
| 21 | PA7 | LIMIT_SW4 | GPIO_Input | IN | PULLUP | Axis 2 limit switch 2 — active LOW (normally open, pulled high) |
| MODBUS NODE ADDRESS & CONFIGURATION · Right side, pins 42-45 |  |  |  |  |  |  |
| 42 | PA8 | CONFIG_SW | GPIO_Input | IN | PULLUP | General-purpose configuration switch — active LOW |
| 45 | PA11 | ADDR0 | GPIO_Input | IN | PULLUP | Modbus node address bit 0 |
| 44 | PA10 | ADDR1 | GPIO_Input | IN | PULLUP | Modbus node address bit 1 |
| 43 | PA9 | ADDR2 | GPIO_Input | IN | PULLUP | Modbus node address bit 2 — 3-bit addressing supports 8 node addresses |
| FAN CONTROL · Left side, pin 2 |  |  |  |  |  |  |
| 2 | PC13 | FAN | GPIO_Output | OUT | — | Fan 12 V on/off — drives AO3400A MOSFET gate to switch 12 V fan supply |
| STATUS LED · Right side, pin 46 |  |  |  |  |  |  |
| 46 | PA12 | STATUS_LED | GPIO_Output | OUT | — | Status LED (active HIGH — drives LED via current-limit resistor) |
| DEBUG & PROGRAMMING (SWD + SWO) · Top side, pins 49, 55, 64 |  |  |  |  |  |  |
| 49 | PA13 | SWDIO | SYS_JTMS-SWDIO | I/O | — | SWD data — 2-wire programming/debug interface |
| 64 | PA14 | SWCLK | SYS_JTCK-SWCLK | IN | — | SWD clock |
| 55 | PB3 | SWO | SYS_JTDO-SWO | OUT | — | Single-wire trace output (ITM/ETM) — PB3 is the only SWO pin on STM32G474 |
| HSE CRYSTAL — 16 MHz XTAL → 170 MHz SYSCLK · Left side, pins 5-6 |  |  |  |  |  |  |
| 5 | PF0 | OSC_IN | RCC_HSE_IN | IN | — | HSE crystal input (16 MHz) — PLL M=4, N=85, R=2 → 170 MHz SYSCLK |
| 6 | PF1 | OSC_OUT | RCC_HSE_OUT | OUT | — | HSE crystal output |
| BOOT0 — RESERVED · Top side, pin 60 |  |  |  |  |  |  |
| 60 | PB8 | BOOT0 | BOOT0 (dedicated) | IN | PULLDOWN | Boot mode — pull LOW via 10 kΩ for normal boot. Reserved — do not share with GPIO. |
| DEDICATED & POWER PINS |  |  |  |  |  |  |
| 7 | PG10 | NRST | RESET (dedicated) | IN | — | Hardware reset — active LOW. 100 nF cap to GND. Expose on SWD connector. |
| 1 | — | VBAT | Power | — | — | Backup supply for RTC (1.71–3.6 V). Connect to VDD + 100 nF cap to GND. |
| 27 | — | VSSA | GND | — | — | Analog ground — connect to GND plane |
| 28 | — | VREF+ | Power | — | — | ADC/DAC reference — connect to VDDA with LC filter if ADC used |
| 29 | — | VDDA | Power | — | — | Analog supply — 1 µF + 10 nF decoupling to VSSA |
| 15 | — | VSS | GND | — | — | Digital ground |
| 16 | — | VDD | Power | — | — | Digital supply |
| 31 | — | VSS | GND | — | — | Digital ground |
| 32 | — | VDD | Power | — | — | Digital supply |
| 47 | — | VSS | GND | — | — | Digital ground |
| 48 | — | VDD | Power | — | — | Digital supply |
| 62 | — | VSS | GND | — | — | Digital ground |
| 63 | — | VDD | Power | — | — | Digital supply |
| UNUSED I/O PINS — 10 available for future expansion |  |  |  |  |  |  |
| 3 | PC14 | — | — | — | — | Free. LSE not populated. OSC32_IN AF. |
| 4 | PC15 | — | — | — | — | Free. LSE not populated. OSC32_OUT AF. |
| 11 | PC3 | — | — | — | — | Free. Left side. TIM1_CH4, SAI1_SD_A AFs. |
| 22 | PC4 | — | — | — | — | Free. Bottom. USART1_TX, I2C2_SCL AFs. |
| 23 | PC5 | — | — | — | — | Free. Bottom. USART1_RX AF. |
| 50 | PA15 | — | — | — | — | Free. Top side. TIM2_CH1, SPI1_NSS, USART2_RX AFs. |
| 53 | PC12 | — | — | — | — | Free. Top side. USART3_CK, SPI3_MOSI AFs. |
| 56 | PB4 | — | — | — | — | Free. Top side. TIM3_CH1, SPI1_MISO, USART2_RX AFs. |

## Design Notes

- Package: STM32G474RET6, LQFP64 (10×10 mm).

- Dark red = new or reassigned. Dark green = unused/available. Grey = power or dedicated.

- Rev 7 changes from Rev 6: (1) I2C3 replaced by I2C4 on PC6/PC7 — I2C4_SCL=PC6, I2C4_SDA=PC7. PC6 is the only available I2C4_SCL pin on LQFP64. (2) CONFIG_SW moved to PA8 (freed from I2C3_SCL). (3) Full second axis added: encoder on TIM1/PC0/PC1/PC2, motor on TIM8/PC8/PC9/PA5, limit switches on PA6/PA7.

- I2C4 is functionally identical to I2C3 for this application — same speed modes, same HAL API (HAL_I2C_*), same APB1 bus. LM75B, LTC4311, and M24C64 are unaffected. Timing register value is identical.

- Axis 2 encoder uses TIM1 in encoder interface mode (identical firmware pattern to TIM2 for axis 1). TIM1 is an advanced control timer with full quadrature decoder support.

- Axis 2 step pulse uses TIM8_CH3 on PC8. TIM8 is an advanced control timer — same PWM capabilities as TIM3. Both axes can run independent speed profiles simultaneously.

- Axis 2 encoder inputs PC0/PC1/PC2 are adjacent to axis 1 encoder inputs PA0/PA1/PA2 on the left side — both encoder connectors can be placed together.

- Axis 2 motor outputs PC8/PC9 (PUL2/DIR2) are on the right side. ENABLE2 on PA5 is on the bottom — route to the motor connector as convenient.

- Axis 2 limit switches PA6/PA7 sit adjacent to axis 1 limit switches PA3/PA4 on the bottom side — natural grouping for a shared limit switch header.

- BOOT0 (PB8, pin 60): pull LOW via 10 kΩ to GND for normal boot. Do not share with any signal.

- SWD+SWO debug connector: SWDIO=PA13, SWCLK=PA14, SWO=PB3. Full ITM trace available.

- LPUART1 and USART1/3 are independent peripherals — all three UARTs operate simultaneously.

- 10 free I/O pins remain: PC3, PC4, PC5, PC12, PC14, PC15, PA15, PB4, and 2 others.