# STM32G474RET6 — Axis Controller PCB

## Pin Assignments & Functionality  (Rev 3 — LQFP64)

LQFP-64 package  ·  Dark red = new or reassigned  ·  Dark green = unused / available  ·  Grey = power or dedicated

| Pin # | GPIO | Signal Label | Peripheral / Function | Dir | Pull | Description |
| --- | --- | --- | --- | --- | --- | --- |
| RS-485 — HOST PORT (FOLLOWER / MODBUS SLAVE) · Top side, pins 56-59 |  |  |  |  |  |  |
| 56 | PB4 | RS485_HOST_DE | GPIO_Output | OUT | — | Driver enable for host RS-485 transceiver (HIGH=transmit) — DE pin to ISOW1432B |
| 58 | PB6 | USART1_TX | USART1 | OUT | — | UART TX → host RS-485 transceiver — Modbus RTU to/from host controller |
| 59 | PB7 | USART1_RX | USART1 | IN | — | UART RX ← host RS-485 transceiver |
| RS-485 — LEADER PORT (OUTGOING / FUTURE SERVO CONTROLLER) · Top side, pins 51-53 |  |  |  |  |  |  |
| 51 | PC10 | USART3_TX | USART3 | OUT | — | UART TX → leader RS-485 transceiver — Modbus RTU to downstream servo/device |
| 52 | PC11 | USART3_RX | USART3 | IN | — | UART RX ← leader RS-485 transceiver — response from downstream device |
| 53 | PC12 | RS485_LEAD_DE | GPIO_Output | OUT | — | Driver enable for leader RS-485 transceiver (HIGH=transmit) |
| RS-232 PORT (GPS NMEA via MAX232) · Top side, pins 55-56 [SWO sacrificed for USART2] |  |  |  |  |  |  |
| 55 | PB3 | USART2_TX | USART2 | OUT | — | UART TX → MAX232 level shifter — optional outgoing RS-232 data [PB3 reassigned from SWO] |
| 56 | PB4 | USART2_RX | USART2 | IN | — | UART RX ← MAX232 — GPS NMEA sentence input [shared pin — see note] |
| GPS 1 PPS TIMING INPUT · Top side, pin 61 |  |  |  |  |  |  |
| 61 | PB9 | GPS_1PPS | EXTI9 / TIM input | IN | PULLDOWN | 1 Hz precision timing pulse from GPS unit — sidereal sync / RTC calibration |
| SPI2 — W25Q32 4 MB NOR FLASH · Right side, pins 34-37 (contiguous group) |  |  |  |  |  |  |
| 34 | PB12 | FLASH_CS | GPIO_Output | OUT | — | SPI chip-select for W25Q32 (active LOW) — drive LOW before SPI transaction |
| 35 | PB13 | SPI2_SCK | SPI2 | OUT | — | SPI clock to W25Q32 |
| 36 | PB14 | SPI2_MISO | SPI2 | IN | — | SPI MISO from W25Q32 |
| 37 | PB15 | SPI2_MOSI | SPI2 | OUT | — | SPI MOSI to W25Q32 [reassigned from CONFIG_SW1 in original IOC] |
| I2C3 — SENSOR BUS · Right side, pins 41-42 |  |  |  |  |  |  |
| 42 | PA8 | I2C3_SCL | I2C3 | OUT | — | I2C clock — on-board sensors: LM75B temp, LTC4311 extender, M24C64 EEPROM |
| 41 | PC9 | I2C3_SDA | I2C3 | I/O | — | I2C data — bidirectional sensor bus [moved from PB5 to PC9 for topology] |
| ENCODER (E6B2-CWZ6C) · Left side, pins 12-14 |  |  |  |  |  |  |
| 12 | PA0 | ENC_A | TIM2_CH1 | IN | — | Encoder channel A — quadrature input (TIM2 encoder mode) |
| 13 | PA1 | ENC_B | TIM2_CH2 | IN | — | Encoder channel B — quadrature input (TIM2 encoder mode) |
| 14 | PA2 | ENC_Z | EXTI2 | IN | PULLUP | Encoder index/Z pulse — external interrupt for homing reference |
| DM556Y STEPPER DRIVER · Bottom side, pins 24-26 |  |  |  |  |  |  |
| 24 | PB0 | PUL | TIM3_CH3 PWM | OUT | — | Step pulse output to DM556Y PUL+ (variable-frequency PWM for speed control) |
| 25 | PB1 | DIR | GPIO_Output | OUT | — | Direction signal to DM556Y DIR+ (HIGH=CW, LOW=CCW) |
| 26 | PB2 | ENABLE | GPIO_Output | OUT | — | Motor driver enable — assert to energize motor coils |
| LIMIT SWITCHES · Bottom side, pins 17-18 |  |  |  |  |  |  |
| 17 | PA3 | LIMIT_SW1 | GPIO_Input | IN | PULLUP | Axis limit switch 1 — active LOW (normally open, pulled high) |
| 18 | PA4 | LIMIT_SW2 | GPIO_Input | IN | PULLUP | Axis limit switch 2 — active LOW (normally open, pulled high) |
| MODBUS NODE ADDRESS INPUTS · Left side, pins 8-10 |  |  |  |  |  |  |
| 8 | PC0 | ADDR0 | GPIO_Input | IN | PULLUP | Modbus node address bit 0 — DIP switch / solder jumper |
| 9 | PC1 | ADDR1 | GPIO_Input | IN | PULLUP | Modbus node address bit 1 |
| 10 | PC2 | ADDR2 | GPIO_Input | IN | PULLUP | Modbus node address bit 2 [moved from PA9/PB8 in LQFP48 design] |
| FAN CONTROL · Bottom side, pin 21 |  |  |  |  |  |  |
| 21 | PA7 | FAN | GPIO_Output | OUT | — | Fan 12 V on/off — drives AO3400A MOSFET gate to switch 12 V fan supply |
| STATUS LED · Right side, pin 46 |  |  |  |  |  |  |
| 46 | PA12 | STATUS_LED | GPIO_Output | OUT | — | Status LED output (active HIGH — drives LED via current-limit resistor) |
| RS-485 HOST DRIVER ENABLE (MOVED) · Bottom side, pin 19 |  |  |  |  |  |  |
| 19 | PA5 | RS485_HOST_DE | GPIO_Output | OUT | — | Driver enable for host RS-485 transceiver — moved to PA5 to free PB4 for USART2_RX |
| DEBUG & PROGRAMMING (SWD 2-Wire) · Top side, pins 49 & 64 [SWO not available — PB3 used for USART2_TX] |  |  |  |  |  |  |
| 49 | PA13 | SWDIO | SYS_JTMS-SWDIO | I/O | — | SWD data — 2-wire programming/debug interface |
| 64 | PA14 | SWCLK | SYS_JTCK-SWCLK | IN | — | SWD clock |
| HSE CRYSTAL — 16 MHz XTAL → 170 MHz SYSCLK via PLL · Left side, pins 5-6 |  |  |  |  |  |  |
| 5 | PF0 | OSC_IN | RCC_HSE_IN | IN | — | HSE crystal input (16 MHz) — PLL M=4, N=85, R=2 → 170 MHz SYSCLK |
| 6 | PF1 | OSC_OUT | RCC_HSE_OUT | OUT | — | HSE crystal output |
| BOOT0 — RESERVED · Top side, pin 60 |  |  |  |  |  |  |
| 60 | PB8 | BOOT0 | BOOT0 (dedicated) | IN | PULLDOWN | Boot mode select — sampled at reset. Pull LOW (10 kΩ) for normal boot. Do NOT share with GPIO. |
| DEDICATED & POWER PINS |  |  |  |  |  |  |
| 7 | PG10 | NRST | RESET (dedicated) | IN | — | Hardware reset — active LOW. 100 nF cap to GND. Expose on SWD connector. |
| 1 | — | VBAT | Power | — | — | Backup supply for RTC domain (1.71–3.6 V). Connect to VDD + 100 nF cap. |
| 27 | — | VSSA | GND | — | — | Analog ground — connect directly to GND plane |
| 28 | — | VREF+ | Power | — | — | ADC/DAC reference — connect to VDDA with LC filter if ADC used |
| 29 | — | VDDA | Power | — | — | Analog supply — 1 µF + 10 nF decoupling to VSSA |
| 15 | — | VSS | GND | — | — | Digital ground |
| 16 | — | VDD | Power | — | — | Digital supply (1.71–3.6 V) |
| 31 | — | VSS | GND | — | — | Digital ground |
| 32 | — | VDD | Power | — | — | Digital supply |
| 47 | — | VSS | GND | — | — | Digital ground |
| 48 | — | VDD | Power | — | — | Digital supply |
| 62 | — | VSS | GND | — | — | Digital ground |
| 63 | — | VDD | Power | — | — | Digital supply |
| UNUSED I/O PINS — 14 available for future expansion |  |  |  |  |  |  |
| 2 | PC13 | — | — | — | — | Free. Has TIM1_BKIN, TIM1_CH1N AFs. Low-speed pin — avoid high-frequency use. |
| 3 | PC14 | — | — | — | — | Free. LSE crystal NOT populated. Has OSC32_IN AF. |
| 4 | PC15 | — | — | — | — | Free. LSE crystal NOT populated. Has OSC32_OUT AF. |
| 11 | PC3 | — | — | — | — | Free. Bottom-left. Has USART2_TX, SPI2_MOSI AFs. |
| 20 | PA6 | — | — | — | — | Free. Bottom. Has TIM3_CH1, SPI1_MISO AFs. |
| 22 | PC4 | — | — | — | — | Free. Bottom. Has USART1_TX, I2C2_SCL AFs. |
| 23 | PC5 | — | — | — | — | Free. Bottom. Has USART1_RX AFs. |
| 30 | PB10 | — | — | — | — | Free. Bottom. Has USART3_TX, LPUART1_TX, TIM2_CH3 AFs. Good UART/timer expansion pin. |
| 33 | PB11 | — | — | — | — | Free. Right side. Has TIM2_CH4, SPI2_NSS AFs. |
| 38 | PC6 | — | — | — | — | Free. Right side. Has TIM3_CH1, TIM8_CH1, I2C4_SCL AFs. |
| 39 | PC7 | — | — | — | — | Free. Right side. Has TIM3_CH2, TIM8_CH2, I2C4_SDA AFs. |
| 40 | PC8 | — | — | — | — | Free. Right side. Has TIM3_CH3, TIM8_CH3, I2C3_SCL AFs. |
| 43 | PA9 | — | — | — | — | Free. Right side. Has USART1_TX, TIM1_CH2, I2C2_SCL AFs. |
| 54 | PD2 | — | — | — | — | Free. Top side. Has TIM3_ETR, TIM8_BKIN AFs. |

## Design Notes

- Package: STM32G474RET6, LQFP64 (10×10 mm). Upgraded from LQFP48 to resolve USART2/USART3 pin availability constraints.

- Dark red pins are new or reassigned relative to the original LQFP48 IOC baseline. Dark green pins are unassigned and available.

- SWO trace output is NOT available in this design — PB3 is reassigned to USART2_TX (RS-232). Standard 2-wire SWD (SWDIO + SWCLK) is retained for programming and debugging.

- PB4 serves dual purpose: it is the USART2_RX pin for RS-232 input. RS485_HOST_DE has been moved to PA5 to free PB4 for this function.

- USART3 (PC10/PC11) is now cleanly available on LQFP64 — this was the primary reason for the package upgrade from LQFP48.

- I2C3_SDA moved from PB5 (LQFP48) to PC9 (LQFP64) for better right-side topology alignment with the I2C sensor connector.

- Address inputs ADDR0-2 moved to PC0-PC2 (left side). These were scattered across PA9/PA10/PA11 in the LQFP48 design.

- SPI2 flash group PB12-PB15 is contiguous on the right side — clean routing to W25Q32.

- BOOT0 (PB8, pin 60) is reserved exclusively. Pull LOW via 10 kΩ resistor to GND for normal operation. Do not share with any signal.

- HSE 16 MHz crystal on PF0/PF1. PLL: M=4, N=85, R=2 → 170 MHz SYSCLK. Flash latency WS=4 required at this frequency.

- LSE 32.768 kHz crystal (PC14/PC15) is NOT populated. PC14 and PC15 are available as free GPIO.

- VBAT (pin 1) should be connected to VDD with a 100 nF ceramic decoupling cap, or to a 3 V coin cell for RTC backup retention.

- 14 unused I/O pins remain available. Notable expansion candidates: PB10 (USART3_TX/LPUART1_TX), PC3 (USART2_TX), PA9 (USART1_TX), PC6-PC8 (TIM3/TIM8/I2C).