# STM32G474RET6 — Axis Controller PCB

## Pin Assignments & Functionality  (Rev 4 — LQFP64)

LQFP-64  ·  Dark red = new/reassigned  ·  Dark green = unused/available  ·  Grey = power/dedicated

| Pin # | GPIO | Signal Label | Peripheral / Function | Dir | Pull | Description |
| --- | --- | --- | --- | --- | --- | --- |
| RS-485 — HOST PORT (FOLLOWER / MODBUS SLAVE) · Top side, pins 57-59 [grouped PB5/PB6/PB7] |  |  |  |  |  |  |
| 57 | PB5 | RS485_HOST_DE | GPIO_Output | OUT | — | Driver enable for host RS-485 transceiver (HIGH=transmit) — grouped with USART1 TX/RX pins |
| 58 | PB6 | USART1_TX | USART1 | OUT | — | UART TX → host RS-485 transceiver — Modbus RTU to host controller |
| 59 | PB7 | USART1_RX | USART1 | IN | — | UART RX ← host RS-485 transceiver — Modbus RTU from host |
| RS-485 — LEADER PORT (OUTGOING / FUTURE SERVO CONTROLLER) · Top side, pins 51-55 |  |  |  |  |  |  |
| 51 | PC10 | USART3_TX | USART3 | OUT | — | UART TX → leader RS-485 transceiver — Modbus RTU to downstream servo/device |
| 52 | PC11 | USART3_RX | USART3 | IN | — | UART RX ← leader RS-485 transceiver — response from downstream device |
| 53 | PC12 | RS485_LEAD_DE_old | — | — | — | NOTE: see PD2 — RS485_LEAD_DE moved here from PC12 per rev4 change |
| 55 | PD2 | RS485_LEAD_DE | GPIO_Output | OUT | — | Driver enable for leader RS-485 transceiver (HIGH=transmit) [moved from PC12 to PD2] |
| RS-232 PORT (GPS NMEA via MAX232) · LPUART1 on PB10/PC0 [USART2 freed; SWO restored on PB3] |  |  |  |  |  |  |
| 30 | PB10 | LPUART1_TX | LPUART1 | OUT | — | LPUART1 TX → MAX232 level shifter — RS-232 outgoing data to GPS or other device |
| 8 | PC0 | LPUART1_RX | LPUART1 | IN | — | LPUART1 RX ← MAX232 — GPS NMEA sentence input [ADDR0 moved to PA9 to free this pin] |
| GPS 1 PPS TIMING INPUT · Top side, pin 61 |  |  |  |  |  |  |
| 61 | PB9 | GPS_1PPS | EXTI9 / TIM input | IN | PULLDOWN | 1 Hz precision timing pulse from GPS — sidereal sync / RTC calibration |
| SPI2 — W25Q32 4 MB NOR FLASH · Right side, pins 34-37 (contiguous group PB12-PB15) |  |  |  |  |  |  |
| 34 | PB12 | FLASH_CS | GPIO_Output | OUT | — | SPI chip-select for W25Q32 (active LOW) |
| 35 | PB13 | SPI2_SCK | SPI2 | OUT | — | SPI clock to W25Q32 |
| 36 | PB14 | SPI2_MISO | SPI2 | IN | — | SPI MISO from W25Q32 |
| 37 | PB15 | SPI2_MOSI | SPI2 | OUT | — | SPI MOSI to W25Q32 |
| I2C3 — SENSOR BUS · Right side, pins 41-42 |  |  |  |  |  |  |
| 42 | PA8 | I2C3_SCL | I2C3 | OUT | — | I2C clock — on-board sensors: LM75B temp, LTC4311 extender, M24C64 EEPROM |
| 41 | PC9 | I2C3_SDA | I2C3 | I/O | — | I2C data — bidirectional sensor bus |
| ENCODER (E6B2-CWZ6C) · Left side, pins 12-14 |  |  |  |  |  |  |
| 12 | PA0 | ENC_A | TIM2_CH1 | IN | — | Encoder channel A — quadrature input (TIM2 encoder mode) |
| 13 | PA1 | ENC_B | TIM2_CH2 | IN | — | Encoder channel B — quadrature input (TIM2 encoder mode) |
| 14 | PA2 | ENC_Z | EXTI2 | IN | PULLUP | Encoder index/Z pulse — external interrupt for homing reference |
| DM556Y STEPPER DRIVER · Bottom side, pins 24-26 |  |  |  |  |  |  |
| 24 | PB0 | PUL | TIM3_CH3 PWM | OUT | — | Step pulse to DM556Y PUL+ (variable-frequency PWM for speed control) |
| 25 | PB1 | DIR | GPIO_Output | OUT | — | Direction signal to DM556Y DIR+ (HIGH=CW, LOW=CCW) |
| 26 | PB2 | ENABLE | GPIO_Output | OUT | — | Motor driver enable — assert to energize motor coils |
| LIMIT & CONFIGURATION SWITCHES · Bottom/left, pins 17-18 & 11 |  |  |  |  |  |  |
| 17 | PA3 | LIMIT_SW1 | GPIO_Input | IN | PULLUP | Axis limit switch 1 — active LOW (normally open, pulled high) |
| 18 | PA4 | LIMIT_SW2 | GPIO_Input | IN | PULLUP | Axis limit switch 2 — active LOW (normally open, pulled high) |
| 11 | PC3 | CONFIG_SW | GPIO_Input | IN | PULLUP | General-purpose configuration switch — active LOW [new, left side] |
| MODBUS NODE ADDRESS INPUTS · Left/right, pins 9-10 & 43 |  |  |  |  |  |  |
| 43 | PA9 | ADDR0 | GPIO_Input | IN | PULLUP | Modbus node address bit 0 [moved from PC0 to PA9 — PC0 freed for LPUART1_RX] |
| 9 | PC1 | ADDR1 | GPIO_Input | IN | PULLUP | Modbus node address bit 1 |
| 10 | PC2 | ADDR2 | GPIO_Input | IN | PULLUP | Modbus node address bit 2 |
| FAN CONTROL · Left side, pin 2 [moved from PA7 to PC13] |  |  |  |  |  |  |
| 2 | PC13 | FAN | GPIO_Output | OUT | — | Fan 12 V on/off — drives AO3400A MOSFET gate [moved from PA7 to PC13 per rev4] |
| STATUS LED · Right side, pin 46 |  |  |  |  |  |  |
| 46 | PA12 | STATUS_LED | GPIO_Output | OUT | — | Status LED output (active HIGH — drives LED via current-limit resistor) |
| DEBUG & PROGRAMMING (SWD + SWO) · Top side, pins 49, 55, 64 |  |  |  |  |  |  |
| 49 | PA13 | SWDIO | SYS_JTMS-SWDIO | I/O | — | SWD data — 2-wire programming/debug |
| 64 | PA14 | SWCLK | SYS_JTCK-SWCLK | IN | — | SWD clock |
| 55 | PB3 | SWO | SYS_JTDO-SWO | OUT | — | Single-wire trace output (ITM/ETM) [restored — RS-232 moved to LPUART1, freeing PB3] |
| HSE CRYSTAL — 16 MHz XTAL → 170 MHz SYSCLK · Left side, pins 5-6 |  |  |  |  |  |  |
| 5 | PF0 | OSC_IN | RCC_HSE_IN | IN | — | HSE crystal input (16 MHz) — PLL M=4, N=85, R=2 → 170 MHz SYSCLK |
| 6 | PF1 | OSC_OUT | RCC_HSE_OUT | OUT | — | HSE crystal output |
| BOOT0 — RESERVED · Top side, pin 60 |  |  |  |  |  |  |
| 60 | PB8 | BOOT0 | BOOT0 (dedicated) | IN | PULLDOWN | Boot mode select — sampled at reset. Pull LOW via 10 kΩ to GND for normal boot. Reserved — do not share. |
| DEDICATED & POWER PINS |  |  |  |  |  |  |
| 7 | PG10 | NRST | RESET (dedicated) | IN | — | Hardware reset — active LOW. 100 nF cap to GND. Expose on SWD connector. |
| 1 | — | VBAT | Power | — | — | Backup supply for RTC (1.71–3.6 V). Connect to VDD + 100 nF cap to GND. |
| 27 | — | VSSA | GND | — | — | Analog ground — connect directly to GND plane |
| 28 | — | VREF+ | Power | — | — | ADC/DAC reference — connect to VDDA with LC filter if ADC used |
| 29 | — | VDDA | Power | — | — | Analog supply — 1 µF + 10 nF decoupling to VSSA |
| 15 | — | VSS | GND | — | — | Digital ground |
| 16 | — | VDD | Power | — | — | Digital supply |
| 31 | — | VSS | GND | — | — | Digital ground |
| 32 | — | VDD | Power | — | — | Digital supply |
| 47 | — | VSS | GND | — | — | Digital ground |
| 48 | — | VDD | Power | — | — | Digital supply |
| 62 | — | VSS | GND | — | — | Digital ground |
| 63 | — | VDD | Power | — | — | Digital supply |
| UNUSED I/O PINS — 13 available for future expansion |  |  |  |  |  |  |
| 3 | PC14 | — | — | — | — | Free. LSE not populated. OSC32_IN AF available. |
| 4 | PC15 | — | — | — | — | Free. LSE not populated. OSC32_OUT AF available. |
| 19 | PA5 | — | — | — | — | Free. Bottom. SPI1_SCK, USART2_CK AFs. |
| 20 | PA6 | — | — | — | — | Free. Bottom. TIM3_CH1, SPI1_MISO AFs. |
| 21 | PA7 | — | — | — | — | Free. Bottom. TIM3_CH2, SPI1_MOSI AFs. [freed from FAN — FAN moved to PC13] |
| 22 | PC4 | — | — | — | — | Free. Bottom. USART1_TX, I2C2_SCL AFs. |
| 23 | PC5 | — | — | — | — | Free. Bottom. USART1_RX AF. |
| 33 | PB11 | — | — | — | — | Free. Right side. TIM2_CH4, SPI2_NSS AFs. |
| 38 | PC6 | — | — | — | — | Free. Right side. TIM3_CH1, TIM8_CH1, I2C4_SCL AFs. |
| 39 | PC7 | — | — | — | — | Free. Right side. TIM3_CH2, TIM8_CH2, I2C4_SDA AFs. |
| 40 | PC8 | — | — | — | — | Free. Right side. TIM3_CH3, TIM8_CH3, I2C3_SCL AFs. |
| 44 | PA10 | — | — | — | — | Free. Right side. USART1_RX, TIM1_CH3, I2C2_SDA AFs. |
| 56 | PB4 | — | — | — | — | Free. Top side. TIM3_CH1, SPI1_MISO, USART2_RX AFs. [freed when RS-232 moved to LPUART1] |

## Design Notes

- Package: STM32G474RET6, LQFP64 (10×10 mm).

- Dark red = new or reassigned. Dark green = unused/available. Grey = power or dedicated function.

- Rev 4 changes from Rev 3: (1) RS485_HOST_DE moved PB5→grouped with PB6/PB7. (2) RS485_LEAD_DE moved PC12→PD2. (3) FAN moved PA7→PC13. (4) CONFIG_SW added on PC3. (5) SWO restored on PB3 by moving RS-232 from USART2 to LPUART1.

- SWO (PB3) is restored. PB3 is the only pin on the STM32G474 that carries JTDO-TRACESWO — there is no alternate. Moving RS-232 to LPUART1 was necessary to free PB3.

- RS-232 / GPS NMEA uses LPUART1: TX=PB10 (pin 30, bottom), RX=PC0 (pin 8, left). LPUART1 fully supports GPS NMEA baud rates (4800, 9600, 38400). USART2 is now unused.

- ADDR0 moved from PC0 to PA9 (pin 43, right side) to free PC0 for LPUART1_RX.

- PC12 is now free (RS485_LEAD_DE moved to PD2). PA7 is now free (FAN moved to PC13). PB4 is now free (RS-232 moved to LPUART1).

- CONFIG_SW on PC3 (pin 11, left side) — active LOW, internal pull-up. Provides a 4th switch alongside LIMIT_SW1, LIMIT_SW2, and BOOT0.

- BOOT0 (PB8, pin 60): pull LOW via 10 kΩ to GND for normal boot. BOOT0 is sampled at reset — do not let any signal drive it HIGH inadvertently.

- SPI2 flash group PB12–PB15 (pins 34–37) is contiguous on the right side — clean routing to W25Q32.

- I2C3_SDA on PC9 (right side) — confirmed AF on LQFP64. PA8/PC9 are adjacent right-side pins, minimising I2C trace length.

- HSE 16 MHz crystal on PF0/PF1. PLL: PLLM=4, PLLN=85, PLLR=2 → 170 MHz. Flash wait states = WS4 (set automatically by HAL).

- 13 free I/O pins. Best expansion candidates: PB4 (USART2_RX/TIM3_CH1), PA10 (USART1_RX), PC4/PC5 (USART1_TX/RX), PC6-PC8 (TIM3/TIM8/I2C).